STMS32H743 Drone Controller Board

This board was designed (components selection, schematics drawing and PCB drawing) during 1 weekend.

Board consists of STM32H743VIT, DCDC circuit based on LT8609SEVPBF ultra LOW-EMI SMPS, with optimized components placement and power path (hot loop) to maintain best EMC compatibility and circuit efficiency.

It also contains W25Q512JVEIQ flash with proper routing, and BOSCH BMI088 IMU for enviromental sensory.

Board uses 4 layer stackup, where
1 –  TOP is signals mixed with GND

2 – GND PLANE for low inductance forward currents and EMC

3 – POWER PLANE for low inductance power and EMC

4 – BOTTOM SIGNALS mixed with GND