STM32H743 Drone Controller Board

This board was designed including component selection, schematic capture and PCB layout during one weekend.

The board is based on the STM32H743VIT6 and contains a DC/DC power supply based on the LT8609SEV#PBF ultra-low-EMI SMPS, with optimized component placement, short high-current paths and a minimized switching hot loop to improve EMC performance, reduce parasitic inductance and maintain high power efficiency.

It also contains a W25Q512JVEIQ Flash memory with short high-speed routing, and a Bosch BMI088 IMU for inertial sensing using an integrated accelerometer and gyroscope.

The board uses a 4-layer stackup, where:
1 – TOP: signals, components, local GND copper and power routing
2 – GND PLANE: solid ground plane providing low-inductance return current paths, good signal reference and improved EMC performance
3 – POWER PLANE: low-impedance power distribution plane
4 – BOTTOM: secondary signal routing mixed with GND copper